#include "ns3/core-module.h"

#include "ns3/network-module.h"

#include "ns3/mobility-module.h"

#include "ns3/wifi-module.h"

#include "ns3/internet-module.h"

#include "ns3/applications-module.h"

#include "ns3/netanim-module.h"

#include "ns3/flow-monitor-module.h"

#include <iostream>

#include <vector>

using namespace ns3;

NS_LOG_COMPONENT_DEFINE("UAV-WSN-Collection");

// Global variables to track data collection

uint32_t packetsCollected = 0;

std::vector<uint32_t> sensorDataSent;

// Function called when sensor sends data

void PacketSentCallback(std::string context, Ptr<const Packet> packet) {

static uint32_t lastUavPosition = 0;

// This would be expanded to track which sensor sent data

NS_LOG_UNCOND("📡 Sensor sent " << packet->GetSize() << " bytes");

}

// Function called when UAV receives data

void PacketReceivedCallback(std::string context, Ptr<const Packet> packet, const Address &address) {

packetsCollected++;

NS_LOG_UNCOND("🚁 UAV received packet #" << packetsCollected << " ("

<< packet->GetSize() << " bytes) at time "

<< Simulator::Now().GetSeconds() << "s");

}

int main(int argc, char *argv[]) {

// Enable logging

LogComponentEnable("UAV-WSN-Collection", LOG_LEVEL_INFO);

// Default parameters

uint32_t numSensorNodes = 10;

double totalTime = 100.0;        // seconds

double uavSpeed = 5.0;            // m/s

double collectionRange = 50.0;     // meters

bool enableAnimation = true;

// Parse command line arguments

CommandLine cmd;

cmd.AddValue("numSensors", "Number of sensor nodes", numSensorNodes);

cmd.AddValue("time", "Total simulation time", totalTime);

cmd.AddValue("uavSpeed", "UAV speed (m/s)", uavSpeed);

cmd.AddValue("range", "Collection range (meters)", collectionRange);

cmd.AddValue("animate", "Enable NetAnim animation", enableAnimation);

cmd.Parse(argc, argv);

NS_LOG_UNCOND("🚀 Starting UAV WSN Collection Simulation");

NS_LOG_UNCOND("📊 Configuration: " << numSensorNodes << " sensors, "

<< totalTime << "s duration, UAV speed " << uavSpeed << " m/s");

// Create nodes: [0] = UAV, [1...numSensors] = sensor nodes

NodeContainer uavNode;

uavNode.Create(1);

NodeContainer sensorNodes;

sensorNodes.Create(numSensorNodes);

NodeContainer allNodes = NodeContainer(uavNode, sensorNodes);

// ===== 1. SETUP WIFI (Ad-hoc mode for both UAV and sensors) =====

WifiHelper wifi;

wifi.SetStandard(WIFI_STANDARD_80211g);

WifiMacHelper mac;

mac.SetType("ns3::AdhocWifiMac");  // Ad-hoc mode for direct communication

YansWifiPhyHelper phy;

YansWifiChannelHelper channel = YansWifiChannelHelper::Default();

phy.SetChannel(channel.Create());

// Install WiFi on all nodes

NetDeviceContainer allDevices = wifi.Install(phy, mac, allNodes);

// ===== 2. SETUP MOBILITY =====

MobilityHelper mobility;

// UAV Mobility (Waypoint model)

Ptr<ListPositionAllocator> uavPositionAlloc = CreateObject<ListPositionAllocator>();

uavPositionAlloc->Add(Vector(0.0, 0.0, 50.0));  // Start at (0,0) at 50m altitude

mobility.SetPositionAllocator(uavPositionAlloc);

mobility.SetMobilityModel("ns3::WaypointMobilityModel");

mobility.Install(uavNode);

// Define UAV waypoints (flying in a grid pattern)

Ptr<WaypointMobilityModel> uavMobility = uavNode.Get(0)->GetObject<WaypointMobilityModel>();

double waypointTime = 0.0;

double gridSize = 150.0;  // meters

// Create a lawnmower pattern

for (int i = 0; i < 5; i++) {

// Move right

waypointTime += gridSize / uavSpeed;

uavMobility->AddWaypoint(Waypoint(Seconds(waypointTime), Vector(gridSize, i * 30, 50.0)));

// Move up

waypointTime += 30.0 / uavSpeed;

uavMobility->AddWaypoint(Waypoint(Seconds(waypointTime), Vector(gridSize, (i + 1) * 30, 50.0)));

// Move left

waypointTime += gridSize / uavSpeed;

uavMobility->AddWaypoint(Waypoint(Seconds(waypointTime), Vector(0.0, (i + 1) * 30, 50.0)));

// Move up (if not last)

if (i < 4) {

waypointTime += 30.0 / uavSpeed;

uavMobility->AddWaypoint(Waypoint(Seconds(waypointTime), Vector(0.0, (i + 2) * 30, 50.0)));

}

}

// Sensor Nodes Mobility (Static)

Ptr<ListPositionAllocator> sensorPositionAlloc = CreateObject<ListPositionAllocator>();

// Place sensors in a grid pattern on ground (z=0)

for (uint32_t i = 0; i < numSensorNodes; i++) {

double x = (i % 5) * 40.0;  // Spread out sensors

double y = (i / 5) * 40.0;

sensorPositionAlloc->Add(Vector(x, y, 0.0));

}

mobility.SetPositionAllocator(sensorPositionAlloc);

mobility.SetMobilityModel("ns3::ConstantPositionMobilityModel");

mobility.Install(sensorNodes);

// ===== 3. INSTALL INTERNET STACK =====

InternetStackHelper internet;

internet.Install(allNodes);

Ipv4AddressHelper ipv4;

ipv4.SetBase("10.1.1.0", "255.255.255.0");

Ipv4InterfaceContainer interfaces = ipv4.Assign(allDevices);

// ===== 4. SETUP APPLICATIONS =====

// Create a packet sink on UAV to receive data

uint16_t port = 8080;

Address uavAddress(InetSocketAddress(interfaces.GetAddress(0), port));

PacketSinkHelper packetSinkHelper("ns3::UdpSocketFactory", uavAddress);

ApplicationContainer sinkApp = packetSinkHelper.Install(uavNode.Get(0));

sinkApp.Start(Seconds(1.0));

sinkApp.Stop(Seconds(totalTime));

// Install data generators on sensor nodes

for (uint32_t i = 1; i <= numSensorNodes; i++) {

// Each sensor sends UDP packets to UAV

UdpClientHelper client(interfaces.GetAddress(0), port);

client.SetAttribute("MaxPackets", UintegerValue(100));

client.SetAttribute("Interval", TimeValue(Seconds(5.0)));  // Send every 5 seconds

client.SetAttribute("PacketSize", UintegerValue(64));

ApplicationContainer clientApp = client.Install(sensorNodes.Get(i-1));

// Stagger start times to avoid collisions

clientApp.Start(Seconds(2.0 + i * 0.5));

clientApp.Stop(Seconds(totalTime - 1.0));

}

// ===== 5. TRACING AND CALLBACKS =====

// Trace packet reception at UAV

Config::Connect("/NodeList/0/ApplicationList/*/$ns3::PacketSink/Rx",

MakeCallback(&PacketReceivedCallback));

// ===== 6. ENABLE PCAP TRACING =====

phy.EnablePcap("wsn-uav", allDevices);

// ===== 7. NETANIM ANIMATION =====

AnimationInterface anim("wsn-uav-collection.xml");

if (enableAnimation) {

// Set colors and descriptions

anim.UpdateNodeDescription(uavNode.Get(0), "UAV");

anim.UpdateNodeColor(uavNode.Get(0), 255, 0, 0);  // Red for UAV

for (uint32_t i = 0; i < numSensorNodes; i++) {

std::string desc = "Sensor-" + std::to_string(i+1);

anim.UpdateNodeDescription(sensorNodes.Get(i), desc);

anim.UpdateNodeColor(sensorNodes.Get(i), 0, 255, 0);  // Green for sensors

}

// Set animation boundaries

anim.SetBoundary(0, 0, 200, 200);

}

// ===== 8. FLOW MONITOR FOR STATISTICS =====

FlowMonitorHelper flowHelper;

Ptr<FlowMonitor> monitor = flowHelper.InstallAll();

// ===== 9. RUN SIMULATION =====

NS_LOG_UNCOND("▶️ Starting simulation...");

Simulator::Stop(Seconds(totalTime));

Simulator::Run();

// ===== 10. PRINT STATISTICS =====

monitor->CheckForLostPackets();

Ptr<Ipv4FlowClassifier> classifier = DynamicCast<Ipv4FlowClassifier>(flowHelper.GetClassifier());

FlowMonitor::FlowStatsContainer stats = monitor->GetFlowStats();

uint32_t totalPackets = 0;

uint64_t totalBytes = 0;

for (auto &flow : stats) {

Ipv4FlowClassifier::FiveTuple t = classifier->FindFlow(flow.first);

if (t.destinationAddress == interfaces.GetAddress(0)) {

totalPackets += flow.second.rxPackets;

totalBytes += flow.second.rxBytes;

NS_LOG_UNCOND("Flow " << flow.first << " ("

<< t.sourceAddress << " -> " << t.destinationAddress << "): "

<< flow.second.rxPackets << " packets, "

<< flow.second.rxBytes << " bytes, "

<< "Delay: " << flow.second.delaySum.GetSeconds() / flow.second.rxPackets << "s");

}

}

NS_LOG_UNCOND("✅ Simulation Complete!");

NS_LOG_UNCOND("📊 Summary:");

NS_LOG_UNCOND("   - Total packets collected by UAV: " << packetsCollected);

NS_LOG_UNCOND("   - Total bytes collected: " << totalBytes);

NS_LOG_UNCOND("   - Collection efficiency: "

<< (100.0 * packetsCollected / (numSensorNodes * (totalTime/5))) << "%");

Simulator::Destroy();

if (enableAnimation) {

NS_LOG_UNCOND("🎬 Animation saved to: wsn-uav-collection.xml");

NS_LOG_UNCOND("   View with: netanim wsn-uav-collection.xml");

}

return 0;

}